BLIND MAN GLASS

Ultrasonic Obstacle Detection System for Visually Impaired

# 1.  Experiment Overview

The Blind Man Glass is an assistive wearable prototype designed to help visually impaired individuals detect obstacles in their path. By mounting an HC-SR04 ultrasonic sensor on a spectacle frame and connecting it to an Arduino Nano and active buzzer, the device measures the distance to nearby objects and provides real-time auditory feedback through variable beeping patterns.

## 1.2  Objectives

- Measure the distance of obstacles in front of the user using the HC-SR04 ultrasonic sensor.

- Alert the user through varied buzzer beeping patterns based on obstacle proximity.

- Demonstrate three danger zones: Very Close (0–30 cm), Approaching (31–60 cm), and Caution (61–100 cm).

- Enable the system to run portably using an Arduino Nano as the control unit.

## 1.3  Working Principle

The HC-SR04 sensor emits a 40 kHz ultrasonic pulse from its TRIG pin. When the sound wave hits an obstacle, it reflects back and is detected by the ECHO pin. The Arduino measures the pulse travel time and calculates the distance using:

Distance (cm) = (Echo Duration in µs × 0.034) ÷ 2

The factor 0.034 represents the speed of sound in cm/µs. Dividing by 2 accounts for the round trip (to the object and back).

## 1.4  Buzzer Alert Levels

| S.No | Distance Range | Buzzer Pattern | Indication |
| --- | --- | --- | --- |
| 1 | 0 – 30 cm | Fast Beep (100ms ON / 100ms OFF) | Very Close — Immediate Danger |
| 2 | 31 – 60 cm | Medium Beep (300ms ON / 300ms OFF) | Obstacle Approaching |
| 3 | 61 – 100 cm | Slow Beep (600ms ON / 600ms OFF) | Obstacle Detected — Caution |
| 4 | > 100 cm | Buzzer OFF (Silent) | Path is Clear |

# 2.  Experiment Requirements

## 2.1  Hardware Requirements

- Arduino Nano (ATmega328P based microcontroller)

- HC-SR04 Ultrasonic Distance Sensor

- Active Buzzer (5V compatible)

- Spectacle / Glass Frame (for mounting the sensor)

- Jumper Wires (Male-to-Male and Male-to-Female)

- Mini Breadboard (optional, for prototyping)

- 9V Battery with connector clip (for portable use)

- USB Mini-B Cable (for programming the Arduino Nano)

## 2.2  Software Requirements

- Arduino IDE (version 1.8.x or 2.x) — Free download from arduino.cc

- CH340 USB Driver (for Arduino Nano clones — required on Windows)

- No external libraries required — uses built-in Arduino functions only

# 3.  Components Used

All components listed below are readily available on Robocraze.com — one of India's leading robotics and electronics component stores.

| S.No | Component | Quantity | Buy Link (Robocraze) | price |
| --- | --- | --- | --- | --- |
| 1 | Arduino Nano | 1 | https://robocraze.com/products/nano-development-board-compatible-with-arduino | 210 |
| 2 | HC-SR04 Ultrasonic Sensor | 1 | https://robocraze.com/products/hc-sr-04-ultrasonic-sensor | 76 |
| 3 | Active Buzzer | 1 | https://robu.in/product/5v-passive-buzzer-5pcs/#tab-robu_other_tab | 38 |
| 4 | Jumper wire kit | 1 | https://robocraze.com/products/jumper-wire-set-m2m-m2f-f2f-40-pcs-each | 128 |
| 5 | Mini Breadboard(optional) | 1 | https://robocraze.com/products/mini-breadboard | 20 |
| 6 | 18650 batteries | 2 | https://robocraze.com/products/3-7v-1200mah-18650-battery | 40 |
| 7 | USB Mini-B Cable | 1 | https://robocraze.com/products/mini-usb-cable-0-5-metre-color-may-vary | 48 |
| 8 | Battery case | 1 | https://robocraze.com/products/18650-2-cell-holder | 21 |
| 9 | Construction site specs | 1 | https://amzn.in/d/071bCWrg | 72 |
| 10 | On/off switch | 1 | https://robocraze.com/products/2-pin-mini-on-off-spst-rocker-switch-19-x-13mm | 25 |
|  |  |  | total | 678˜750 |

Note: All links above direct to Robocraze.com. Search the product name on Robocraze if the link redirects to the homepage.

# 4.  Circuit Connectivity

## 4.1  HC-SR04 Ultrasonic Sensor — Arduino Nano

| S.No | Component Pin | Arduino Nano Pin | Description |
| --- | --- | --- | --- |
| 1 | VCC | 5V | Power supply for the sensor |
| 2 | GND | GND | Common ground connection |
| 3 | TRIG | Pin 9 (Digital Output) | Trigger signal — sends ultrasonic pulse |
| 4 | ECHO | Pin 10 (Digital Input) | Echo signal — receives reflected pulse |

## 4.2  Active Buzzer — Arduino Nano

| S.No | Component Pin | Arduino Nano Pin | Description |
| --- | --- | --- | --- |
| 1 | Positive (+) | Pin 11 (Digital Output) | Signal to activate the buzzer |
| 2 | Negative (–) | GND | Common ground connection |

## 4.3  Power Supply

| S.No | Component Pin | Arduino Nano Pin | Description |
| --- | --- | --- | --- |
| 1 | USB Mini-B | PC / Laptop | For uploading code and powering during development |
| 2 | 9V Battery (+) | Vin | For standalone portable operation |
| 3 | 9V Battery (-) | GND Pin | Battery ground |

## 4.4  Circuit Summary Diagram

HC-SR04 Arduino Nano Buzzer VCC ─────► 5V GND ─────► GND ◄───── Buzzer (–) TRIG ─────► D9 ECHO ─────► D10 D11 ─────► Buzzer (+)

# 5.  Step-by-Step Procedure

## Step 1 — Gather All Components

- Collect all components listed in Section 3.

- Install Arduino IDE on your laptop or PC from arduino.cc/en/software.

- Install the CH340 USB driver if you are using a clone Arduino Nano (driver available on GitHub — search CH340 Driver).

## Step 2 — Build the Circuit

- Place the Arduino Nano on the breadboard (if using one).

- Connect HC-SR04 VCC to the Arduino 5V pin using a red jumper wire.

- Connect HC-SR04 GND to the Arduino GND pin using a black jumper wire.

- Connect HC-SR04 TRIG to Arduino Digital Pin 9.

- Connect HC-SR04 ECHO to Arduino Digital Pin 10.

- Connect the Active Buzzer positive (+) terminal to Arduino Digital Pin 11.

- Connect the Active Buzzer negative (–) terminal to Arduino GND.

- Double-check all connections against the circuit diagram in Section 4.

## Step 3 — Upload the Code

- Open Arduino IDE on your computer.

- Connect the Arduino Nano to your PC using the USB Mini-B cable.

- In Arduino IDE, go to Tools > Board and select Arduino Nano.

- Go to Tools > Processor and select ATmega328P (Old Bootloader) if using a clone Nano.

- Go to Tools > Port and select the correct COM port.

- Copy the code from Section 6 and paste it into a new sketch in Arduino IDE.

- Click the Upload button (right-arrow icon) to compile and upload the code.

- Wait for the message 'Done uploading' to appear in the IDE.

## Step 4 — Test the System

- Open Serial Monitor (Ctrl + Shift + M) and set baud rate to 9600.

- Place your hand in front of the HC-SR04 sensor at varying distances.

- Observe the distance readings on the Serial Monitor.

- Verify that the buzzer beeps fast (0–30 cm), medium (31–60 cm), slow (61–100 cm), or stays silent (>100 cm).

- Adjust sensor direction if needed to optimize detection angle.

## Step 5 — Mount on Glass Frame

- Secure the HC-SR04 sensor to the front of the spectacle frame using insulation tape or a small 3D-printed bracket.

- Route the sensor wires along the frame arm neatly, securing with cable ties.

- Place the Arduino Nano and buzzer in a small pocket-sized enclosure or pouch.

- Connect a 9V battery to the Arduino Nano VIN pin for portable operation.

- Wear the glasses and walk around to test obstacle detection in a real-world scenario.

# 6.  Arduino Source Code

The complete, commented source code for the Blind Man Glass project is provided below. No external libraries are required — only built-in Arduino functions are used.

// ============================================================ // Blind Man Glass — Obstacle Detection System // Components: Arduino Nano ¦ HC-SR04 ¦ Active Buzzer // ============================================================ const int trigPin = 9; // HC-SR04 TRIG pin const int echoPin = 10; // HC-SR04 ECHO pin const int buzzerPin = 11; // Active Buzzer positive pin long duration; int distance; void setup() { pinMode(trigPin, OUTPUT); pinMode(echoPin, INPUT); pinMode(buzzerPin, OUTPUT); Serial.begin(9600); } void loop() { // --- Trigger ultrasonic pulse --- digitalWrite(trigPin, LOW); delayMicroseconds(2); digitalWrite(trigPin, HIGH); delayMicroseconds(10); digitalWrite(trigPin, LOW); // --- Read echo & calculate distance --- duration = pulseIn(echoPin, HIGH); distance = duration * 0.034 / 2; // cm Serial.print("Distance: "); Serial.print(distance); Serial.println(" cm"); // --- Buzzer alert logic --- if (distance > 0 && distance <= 30) { digitalWrite(buzzerPin, HIGH); delay(100); // Fast beep digitalWrite(buzzerPin, LOW); delay(100); } else if (distance > 30 && distance <= 60) { digitalWrite(buzzerPin, HIGH); delay(300); // Medium beep digitalWrite(buzzerPin, LOW); delay(300); } else if (distance > 60 && distance <= 100) { digitalWrite(buzzerPin, HIGH); delay(600); // Slow beep digitalWrite(buzzerPin, LOW); delay(600); } else { digitalWrite(buzzerPin, LOW); delay(100); // Silent } }

## 6.1  Code Explanation

- trigPin (9): Sends a 10µs HIGH pulse to trigger the HC-SR04 ultrasonic pulse.

- echoPin (10): Receives the echo signal; pulseIn() measures its duration in microseconds.

- Distance Calculation: distance = duration * 0.034 / 2 converts time to centimeters.

- Buzzer Logic: Three proximity thresholds produce different beeping speeds to communicate danger level.

- Serial.print(): Streams live distance data to the Serial Monitor for debugging.

Circuit Diagram